University of Wisconsin-Madison AWS Specifications

From Technical Manual for Automatic Weather Stations, by George A. Weidner, Department of Meteorology (now Atmospheric and Oceanic Sciences), University of Wisconsin-Madison, 1985.

	Variable
	Sensor
	Specifications

	Air Pressure
	Paroscientific

Model 215 A
	Range: 0 to 1100 hPa

Resolution: 0.050 hPa

Accuracy: +/- 0.2 hPa

(0.2 hPa/year long term drift)

	Air Temperature
	Weed PRT

Two-wire bridge
	Range: to -100 C minimum

Resolution: 0.125 C

Accuracy: +/- 0.5 C

	Humidity
	Vaisala HMP-35A (and other models)
	Range: 0 to 100%

Resolution: 1.0  %

Accuracy: +/- 5.0 % down to -55 C

Corrections possible for lower temperatures

	Wind Direction
	10 K Ohm pot.
	Range: 0 to 355 Degrees

Resolution: 1.5 Degrees  

Accuracy: +/- 3.0 Degrees

	Wind Speed
	Bendix/Belfort

RM Young

Hydro-Tech
	Resolution/Accuracy: 0.25 +/- 0.5 m/s

Resolution/Accuracy: 0.20 +/- 0.5 m/s

Resolution/Accuracy: 0.33 +/- 2%

	Temperature String
	Thermocouple 

Two junction

Copper-Cons.
	Resolution: 0.06 C

Accuracy: +/- 0.125 C


